KLP-740Z

User manual

Please read this user manual before using this product

- Checking before using

| No. | Item | Quantity | Unit |
| --- | --- | --- | --- |
| 1 | Light | 1 | piece |
| 2 | Power cable with blue head | 1 | piece |
| 3 | User manual | 1 | piece |

★For security reasons and in accordance with the terms, reassemble or modify the product is not allowed without permission. Please note that due to human damage caused by improper use of the product, it will break the warranty qualification. Moreover, the unprofessional operation may lead to short circuits, burns or electric shock, and so on.

★This device has been full inspection before shipping. For your safety, please operate it according the User Manual.

★If any damage caused by improper operation will result in the termination of the warranty claims. The manufacturer does not accept any responsibility of property damage due to improper operation or personal injury due to non-compliance with the operating instructions.

★Keep the manual in a safe place for future reference as well as after the product sale, lease, etc.

★Note: Based on product improvements, specification may change without notice. Changes in product specifications related rights will be reserved.

2. Safety Instruction

1. When opening the packing, take all accessories out, and remove the light to a horizontal table for good operation. First check whether the accessories are complete, then check carefully whether the whole light has no damage. If there is any damage, please contact us as soon as possible.

2. AC Power: Check whether the local power supply is accordance with the

requirements of the product rated voltage.

3. The product is only suitable for indoor working environment. The light should

keep dry, keep away from wet, overheat or dusty environment. Don't come into contact with water and other liquids to prevent or reduce the risk of electric shock or fire.

4. Please do not install the light directly on the surface of the combustible substance.

5. The people who install, operate and maintain of light professional certificate required.

6. When there are faults of the light, please stop using the light immediately. Don't try to repair, please contact the nearest authorized technical assistance center, if the parts damage, always use the same type of parts replaced.

7. Don't touch any electrical wiring in the process of operation to prevent the risk of electric shock.

8. To make sure the light working environment temperature, the highest do not exceed 40℃, the lowest do not exceed - 20 ℃.

9. Under the stable cooling working situation, the highest temperature of the housing of the light can reach 80 ℃, please do not touch.

10. The light is design according to the type of electric shock protection. The light should be use in good connection grounding power system and the earth sign port of the light should be connected to the installing fixtures.

11. Do not use the damage of the insulating layer wire and also do not use the power cord overlap on the other wire. When not in use or clean the light, please disconnect the power, don't pull the plug in hard or just drag the wires.

12. There are no components inside for maintenance. Before start operation the light, please check whether all of the shell is installed. In the case of the shell open used the light is prohibited.

Note: Before performing any installation, repair and cleaning of the lamp, please confirm that the power cable has been cut off.

3. Product specification

1.Voltage: AC100-240V, 50-60Hz

2.Power consumption: 300W

3.Light source: 7pcs 40W RGBW 4in1 LED

4.Beam angle: 5-45 °

5.Control mode: voice control, self-propelled, DMX, master / slave.

6.Channel: 5CH,6CH,7CH,9CH,-9CH,10CH,11CH,13CH.

7.Display: TFT full color display

8.Size: 296 * 220 * 436mm

9.Net weight: 7.8kg

4. Display Main Function

| Display Interface |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Main Menu | MENU DOWN UP ENTER | Address | 001-512 | Modify the address code |  |
|  |  |  | 000C-080C | Temperature Display |  |
|  |  | Patterm | 5.6.7.9.-9.10 | Channel Mode Display |  |
|  |  |  | 11,13CH |  |  |
|  |  | Manual control | Red | 000-255 | Red Dimming |
|  |  |  | Green | 000-255 | Green Dimming |
|  |  |  | Blue | 000-255 | Blue Dimming |
|  |  |  | White | 000-255 | White Dimming |
|  |  |  | Strobe | 00HZ--20HZ | Strobe from slow to fast |
|  |  |  | Macro | S-0--S-6 | 1-6,Selection of built-in 6 program |
|  |  |  | Speed | 000-100 | Macro speed Setting |
|  |  |  | Focusing | 000-255 | Focusing manually adjusted |
|  |  | Program | 00-50 | Speed adjustment of Auto-run adjustment |  |
|  |  | Voice | 00-50 | Sensitivity adjustment of voice control program |  |

| Background Operation | In modify address ,press the "Enter"confirmation key 5 seconds to dto enter the background. DOWN--back UP--forward ENTER--determine the exit | Red | 100-255 | Red current setting |
| --- | --- | --- | --- | --- |
|  |  | Green | 100-255 | Green current setting |
|  |  | Blue | 100-255 | Blue current setting |
|  |  | White | 100-255 | White current setting |
|  |  | Patterm | 5.6.7,9,-9 | Channel mode Setting |
|  |  |  | 10.11.13CH |  |
|  |  | Reverse | NO | Macro speed Setting from fast to slow |
|  |  |  | YES | Macro speed Setting from slow to fast |
|  |  | Dimming | TG-0--TG-4 |  |
|  |  | Trip | 000-200 | Focusing Trip Setting |
|  |  | Position | 000-200 | Focusing Position Setting |
|  |  | Send out | NO | Set background values online |
|  |  |  | YES | YES--valid，,press the "Enter" key to send |

When the temperature sensor is installed, the fan starts when the temperature is greater than or equal to 42 degrees Celsius, and stops when the temperature is less than 38 degrees Celsius.Start the intelligent constant temperature function when the temperature is greater than 65 degrees.

5.  Display Function Instruction

6.1,DMX 512 Channel Mode 5 Channel Mode

| CH05 | Channel Function | DMX Value | Function Description |
| --- | --- | --- | --- |
| 5 Channel Mode |  |  |  |
| CH1 | focusing | 0-255 | 0 invalid,1-255 Linear adjustment |
| CH2 | Red | 0-255 | 0-100%W Red Dimming |
| CH3 | Green | 0-255 | 0-100%W Green Dimming |
| CH4 | Blue | 0-255 | 0-100%W Blue Dimming |
| CH5 | White | 0-255 | 0-100%W White Dimming |

6.2,DMX 512 Channel Mode 6 Channel Mode

| CH06 | Channel Function | DMX Value | Function Description |
| --- | --- | --- | --- |
| 6 Channel Mode |  |  |  |
| CH1 | focusing | 0-255 | 0 invalid,1-255 Linear adjustment |
| CH2 | Master Dimming | 0-255 | 0-100% Red、Green、Bluse、White Dimmming |
| CH3 | Red | 0-255 | 0-100%W Red Dimming |
| CH4 | Green | 0-255 | 0-100%W Green Dimming |
| CH5 | Blue | 0-255 | 0-100%W Blue Dimming |
| CH6 | White | 0-255 | 0-100%W White Dimming |

6.3,DMX 512 Channel Mode 7 Channel Mode

| CH07 | Channel Function | DMX Value | Function Description |
| --- | --- | --- | --- |
| 7 Channel Mode |  |  |  |
| CH1 | focusing | 0-255 | 0 invalid,1-255 Linear adjustment |
| CH2 | Master Dimming | 0-255 | 0-100% Red、Green、Bluse、White Dimmming |
| CH3 | Red | 0-255 | 0-100%W Red Dimming |
| CH4 | Green | 0-255 | 0-100%W Green Dimming |
| CH5 | Blue | 0-255 | 0-100%W Blue Dimming |
| CH6 | White | 0-255 | 0-100%W White Dimming |
| CH7 | Strode | 0-255 | 0-12 No operation,13-255，0-20 times/second Strobe from slow to fast |

6.4,DMX 512 Channel Mode 9 Channel Mode(A)

| CH09 | Channel Function | DMX Value | Function Description |
| --- | --- | --- | --- |
| 9 Channel Mode A |  |  |  |
| CH1 | focusing | 0-255 | 0 invalid,1-255 Linear adjustment |
| CH2 | Master Dimming | 0-255 | 0-100% Red、Green、Bluse、White Dimmming |
| CH3 | Red | 0-255 | 0-100%W Red Dimming |
| CH4 | Green | 0-255 | 0-100%W Green Dimming |
| CH5 | Blue | 0-255 | 0-100%W Blue Dimming |
| CH6 | White | 0-255 | 0-100%W White Dimming |
| CH7 | Strode | 0-255 | 0-12 No operation,13-255，0-20 times/second Strobe from slow to fast |
| CH8 | Macro | 0-3 | Invalid |
|  |  | 4-39 | Colors go in slowly and out fastly |
|  |  | 40-75 | Colors go in slowly and out slowly |
|  |  | 76-111 | Colors go in fastly and out slowly |
|  |  | 112-147 | Color, gradient changes |
|  |  | 148-183 | Color,jump changes |
|  |  | 184-219 | Color,Pulse variable |
|  |  | 220-255 | Voice control Funtion |
| CH9 | Macro Speed | 0-255 | Macro Speed，0-100% Speed adjustmentfrom slow to fast |

6.5,DMX 512 Channel Mode 9 Channel Mode(B)

| CH-9 | Channel Function | DMX Value | Function Description |
| --- | --- | --- | --- |
| 9 Channel Mode B |  |  |  |
| CH1 | focusing | 0-255 | 0 invalid,1-255 Linear adjustment |
| CH2 | Master Dimming | 0-255 | 0-100% Red、Green、Bluse、White Dimmming |
| CH3 | Strode | 0-255 | 0-12 No operation,13-255，0-20 times/second Strobe from slow to fast |
| CH4 | Macro | 0-3 | Invalid |
|  |  | 4-39 | Colors go in slowly and out fastly |
|  |  | 40-75 | Colors go in slowly and out slowly |
|  |  | 76-111 | Colors go in fastly and out slowly |
|  |  | 112-147 | Color, gradient changes |
|  |  | 148-183 | Color,jump changes |
|  |  | 184-219 | Color,Pulse variable |
|  |  | 220-255 | Voice control Funtion |
| CH5 | Macro Speed | 0-255 | Macro Speed，0-100% Speed adjustmentfrom slow to fast |
| CH6 | Red | 0-255 | 0-100%W Red Dimming |
| CH7 | Green | 0-255 | 0-100%W Green Dimming |
| CH8 | Blue | 0-255 | 0-100%W Blue Dimming |
| CH9 | White | 0-255 | 0-100%W White Dimming |

6.6,DMX 512 Channel Mode 10 Channel Mode

| CH10 | Channel Function | DMX Value | Function Description |
| --- | --- | --- | --- |
| 10 Channel Mode |  |  |  |
| CH1 | focusing | 0-255 | 0 invalid,1-255 Linear adjustment |
| CH2 | Master Dimming | 0-255 | 0-100% Red、Green、Bluse、White Dimmming |
| CH3 | Red | 0-255 | 0-100%W Red Dimming |
| CH4 | Green | 0-255 | 0-100%W Green Dimming |
| CH5 | Blue | 0-255 | 0-100%W Blue Dimming |
| CH6 | White | 0-255 | 0-100%W White Dimming |
| CH7 | Strode | 0-255 | 0-12 No operation,13-255，0-20 times/second Strobe from slow to fast |
| CH8 | Macro | 0-3 | Invalid |
|  |  | 4-39 | Colors go in slowly and out fastly |
|  |  | 40-75 | Colors go in slowly and out slowly |
|  |  | 76-111 | Colors go in fastly and out slowly |
|  |  | 112-147 | Color, gradient changes |
|  |  | 148-183 | Color,jump changes |
|  |  | 184-219 | Color,Pulse variable |
|  |  | 220-255 | Voice control Funtion |
| CH9 | Macro Speed | 0-255 | Macro Speed，0-100% Speed adjustmentfrom slow to fast |
| CH10 | Color temperature | 0-255 | 0 invalid,1-255,3200-7500K Linear adjustment |
|  | color selector | 0-15 | Invalid |

6.7,DMX 512 Channel Mode 11 Channel Mode

| CH11 | Channel Function | DMX Value | Function Description |
| --- | --- | --- | --- |
| 11 Channel Mode |  |  |  |
| CH1 | focusing | 0-255 | 0 invalid,1-255 Linear adjustment |
| CH2 | Master Dimming | 0-255 | 0-100% Red、Green、Bluse、White Dimmming |
| CH3 | Red | 0-255 | 0-100%W Red Dimming |
| CH4 | Green | 0-255 | 0-100%W Green Dimming |
| CH5 | Blue | 0-255 | 0-100%W Blue Dimming |
| CH6 | White | 0-255 | 0-100%W White Dimming |
| CH7 | Strode | 0-255 | 0-12 No operation,13-255，0-20 times/second Strobe from slow to fast |
| CH8 | Macro | 0-3 | Invalid |
|  |  | 4-39 | Colors go in slowly and out fastly |
|  |  | 40-75 | Colors go in slowly and out slowly |
|  |  | 76-111 | Colors go in fastly and out slowly |
|  |  | 112-147 | Color, gradient changes |
|  |  | 148-183 | Color,jump changes |
|  |  | 184-219 | Color,Pulse variable |
|  |  | 220-255 | Voice control Funtion |
| CH9 | Macro Speed | 0-255 | Macro Speed，0-100% Speed adjustmentfrom slow to fast |
| CH10 | Color temperature | 0-255 | 0 invalid,1-255,3200-7500K Linear adjustment |
| CH11 | color selector | 0-15 | Invalid |
|  |  | 16_31 | R |
|  |  | 32-47 | A |
|  |  | 48-63 | Y1 |
|  |  | 64-79 | Y2 |
|  |  | 80-95 | G |
|  |  | 96-111 | T |
|  |  | 112-127 | C |
|  |  | 128-143 | B |
|  |  | 144-159 | L |
|  |  | 160-175 | M1 |
|  |  | 176-191 | M2 |
|  |  | 192-207 | P |
|  |  | 208-223 | CTO,3200K |
|  |  | 224-239 | CTO,5600K |
|  |  | 240-255 | CTO,7200K |

6.8,DMX 512 Channel Mode 13 Channel Mode

| CH13 | Channel Function | DMX Value | Function Description |
| --- | --- | --- | --- |
| 13 Channel Mode |  |  |  |
| CH1 | focusing | 0-255 | 0 invalid,1-255 Linear adjustment |
| CH2 | Master Dimming | 0-255 | 0-100% Red、Green、Bluse、White Dimmming |
| CH3 | Red | 0-255 | 0-100%W Red Dimming |
| CH4 | Green | 0-255 | 0-100%W Green Dimming |
| CH5 | Blue | 0-255 | 0-100%W Blue Dimming |
| CH6 | White | 0-255 | 0-100%W White Dimming |
| CH7 | Strode | 0-255 | 0-12 No operation,13-255，0-20 times/second Strobe from slow to fast |
| CH8 | Macro | 0-3 | Invalid |
|  |  | 4-39 | Colors go in slowly and out fastly |
|  |  | 40-75 | Colors go in slowly and out slowly |
|  |  | 76-111 | Colors go in fastly and out slowly |
|  |  | 112-147 | Color, gradient changes |
|  |  | 148-183 | Color,jump changes |
|  |  | 184-219 | Color,Pulse variable |
|  |  | 220-255 | Voice control Funtion |
| CH9 | Macro Speed | 0-255 | Macro Speed，0-100% Speed adjustmentfrom slow to fast |
| CH10 | Color temperature | 0-255 | 0 invalid,1-255,3200-7500K Linear adjustment |
| CH11 | color selector | 0-15 | Invalid |
|  |  | 16_31 | R |
|  |  | 32-47 | A |
|  |  | 48-63 | Y1 |
|  |  | 64-79 | Y2 |
|  |  | 80-95 | G |
|  |  | 96-111 | T |
|  |  | 112-127 | C |
|  |  | 128-143 | B |
|  |  | 144-159 | L |
|  |  | 160-175 | M1 |
|  |  | 176-191 | M2 |
|  |  | 192-207 | P |
|  |  | 208-223 | CTO,3200K |
|  |  | 224-239 | CTO,5600K |
|  |  | 240-255 | CTO,7200K |
| CH12 | Color Hue | 0 | Invalid |
|  |  | 1-85 | From Red to Green ,Linear adjustment |
|  |  | 86-170 | From Green to Blue ,Linear adjustment |
|  |  | 171-255 | From Blue to Red ,Linear adjustment |
| CH13 | Color Saturation | 0-255 | 0 invalid，1-255，Linear adjustment from high to low |